IndoorNav: Physically Constrained Multi-Floor Indoor Navigation & 3D Building Digital Twin

Complete Technical Architecture, Physical Collision Constraints, Spatial Graph Extraction, Multi-Floor Routing, and Interactive Three.js Dashboard Documentation

| Institution / Project: | IIT Kharagpur — Digital Twin & Indoor Navigation Pipeline |
| --- | --- |
| System Pipeline: | Phase 1 (3D BIM Model Reconstruction) + Phase 2 (Physically Constrained Graph Navigation) |
| Primary Technologies: | Python (IfcOpenShell, Shapely, NetworkX, NumPy, Trimesh), Three.js WebGL, YOLOv8, OpenCV |
| Date & Status: | September 2026 — Phase 1 & 2 Fully Implemented, Verified, and Tested (8/8 Tests Passing) |

1. Executive Summary & Problem Formulation

Standard Global Positioning System (GPS) signals are rapidly attenuated and scattered by building materials (concrete, steel, and masonry), rendering outdoor satellite navigation completely ineffectual inside multi-storey structures. Indoor navigation requires high-precision topological and geometric representations of the interior architectural environment. Building Information Modeling (BIM), specifically standardized through Industry Foundation Classes (IFC), encapsulates the definitive geometry, semantics, and relationships of architectural spaces, structural walls, doorway portals, and vertical circulation shafts.

The IndoorNav system bridges the gap between raw BIM architectural files and real-time indoor wayfinding. It ingests IFC files, performs 3D geometry and spatial extraction, enforces rigorous physical wall barrier constraints, samples walkable corridor networks, connects multi-floor stairwells and elevators, dynamically re-weights routes based on live CCTV camera crowd feeds, and renders interactive 3D navigation paths inside a WebGL/Three.js building viewer.

NOTE: The Fundamental Physical Invariant A user may move only through explicitly defined navigation nodes and physically valid connections. Walls are absolute physical barriers. Doors and gates are explicit horizontal passage points. Stairs and elevators are explicit vertical passage points between storeys. No shortcuts or wall-crossings are permitted under any geometric condition.

2. End-to-End System Architecture

The IndoorNav pipeline is architected into two seamlessly integrated layers: Phase 1 (BIM Parsing, 3D Reconstruction, and Export) and Phase 2 (Physically Constrained Spatial Graph Building, Dynamic Pathfinding, CCTV AI Crowd Ingestion, and Interactive Dashboard).

2.1 Module Structure & Responsibilities

| Module Path | Layer | Primary Purpose / Output |
| --- | --- | --- |
| src/load_ifc.py | Phase 1 | Loads and validates IFC files via IfcOpenShell; detects schema (IFC2X3/IFC4) and extracts project hierarchy. |
| src/extract_geometry.py | Phase 1 | Extracts triangulated 3D mesh vertices and faces in world coordinates; organizes elements into a storey-keyed scene graph. |
| src/export_model.py | Phase 1 | Transforms Z-up IFC geometry to standard Y-up convention; exports model.obj, model.glb, and lightweight elements.json. |
| src/visualize.py | Phase 1 | PyVista/VTK off-screen rendering engine generating automated color-coded render.png screenshots. |
| src/navigation/collision.py | Phase 2 | Extracts 2D wall boundary footprints, subtracts door openings, and builds Shapely STRtree spatial indices for collision testing. |
| src/navigation/route_validator.py | Phase 2 | Validates candidate graphs for physical constraint violations, wall penetrations, and invalid floor transitions. |
| src/build_graph.py | Phase 2 | Generates explicit multi-floor navigation nodes, samples corridor walking networks, links doors and stairs, exports nav_graph.json. |
| src/navigate.py | Phase 2 | Multi-floor A* and Dijkstra pathfinding engine with dynamic crowd weight penalties and turn-by-turn guidance. |
| src/cctv_integration.py | Phase 2 | Registers physical CCTV cameras in 3D world space, calculates field-of-view viewing cones, and maps cameras to graph nodes. |
| src/crowd_detection.py | Phase 2 | Computer vision pipeline (YOLOv8 nano / OpenCV HOG) providing real-time person counts per camera feed. |
| src/main.py | CLI | Main command-line orchestrator linking stages 1 through 6 with the --build-nav flag. |
| dashboard/index.html | Frontend | Single-file Three.js web application with 3D model loading, graph layer debug toggles, route visualization, collision-safe walk mode, and graph editor. |
| tests/test_navigation.py | Verification | Automated test suite verifying all 8 required navigation, collision, multi-floor, and crowd rerouting scenarios. |

3. Coordinate Systems & Mathematical Conventions

A critical architectural requirement in multi-disciplinary BIM systems is absolute coordinate consistency across the pipeline: raw IFC files, extracted numpy meshes, exported WebGL GLB/OBJ models, NetworkX graph nodes, and the Three.js viewport must remain aligned.

IFC Coordinate System: Follows the civil/architectural standard of Z-up coordinates (X = East/Length, Y = North/Width, Z = Elevation). All vertices extracted by ifcopenshell.geom.create_shape() with USE_WORLD_COORDS=True are preserved in this system.

Graphics / WebGL Standard: WebGL, Three.js, and glTF standards mandate Y-up coordinates (X = Right, Y = Elevation/Up, Z = Depth).

Forward Transformation (IFC → GLB/OBJ): Applied during export in export_model.py: X_glb = X_ifc, Y_glb = Z_ifc, Z_glb = -Y_ifc.

Dashboard Scene Centering: In Three.js, models are centered on the ground plane at (-center_X, -box_min_Y, -center_Z).

Navigation Node Projection: To position an IFC navigation node (x, y, z) into the 3D scene without coordinate drift: X_scene = x + modelGroup.position.x, Y_scene = z + modelGroup.position.y, Z_scene = -y + modelGroup.position.z.

4. Physical Wall Collision & Spatial Layer

To satisfy the non-negotiable requirement that walls must never be traversable, src/navigation/collision.py introduces a physical boundary extraction and collision detection engine.

4.1 2D Planar Footprint Extraction from 3D IFC Meshes

Architectural indoor navigation is fundamentally multi-storey 2D planar navigation interconnected by vertical portals. For each wall entity (IfcWall and IfcWallStandardCase), the 3D mesh is extracted and its triangular faces are projected onto the horizontal XY plane. The exact boundary footprint of the wall is constructed by taking the geometric unary union of the projected 2D triangles:

# Construction of exact 2D wall footprint polygon triangles = [Polygon([verts[face[0]][:2], verts[face[1]][:2], verts[face[2]][:2]]) for face in faces] wall_footprint = shapely.ops.unary_union(triangles)

4.2 Construction of Solid Wall Boundaries

Because doorway openings represent valid physical passages, doors (IfcDoor and IfcOpeningElement) on the same storey have their aperture footprints calculated. The traversable solid wall geometry is defined mathematically as:

# Solid wall geometry excludes authorized door openings SolidWall = WallFootprint.difference(DoorAperturesUnion)

4.3 Segment Raycasting & Penetration Testing

For every candidate edge proposed between node P1 and node P2, a LineString segment is tested against the solid wall STRtree spatial index. If the segment intersects the solid wall polygon with a penetration depth exceeding the tolerance (0.08m), the candidate edge is rejected immediately at graph generation time:

def check_segment_collision(self, p1, p2, storey, penetration_tolerance_m=0.08) -> bool: seg_line = LineString([(p1[0], p1[1]), (p2[0], p2[1])]) candidate_indices = self.wall_trees[storey].query(seg_line) for idx in candidate_indices: solid_poly = self.tree_wall_maps[storey][idx].solid_polygon inter = seg_line.intersection(solid_poly) if inter.length > penetration_tolerance_m: return True # COLLISION DETECTED (BLOCKED) return False # CLEAR (PASS)

5. Multi-Floor Navigation Graph Modeling

The upgraded src/build_graph.py transforms the building model into an explicit, multi-floor topological graph where rooms, corridors, doors, and stairs are distinct node entities.

5.1 Explicit Node Schema & Taxonomy

Every navigation node contains full spatial and level metadata:

{ "id": "node_0BTBFw_01", "type": "corridor", "floor": 1, "storey_id": "Level 1", "space_id": "0BTBFw9p907x1G5lCpt19a", "name": "Corridor A (W1)", "x": 6.42, "y": -8.15, "z": 0.10, "elevation": 0.0, "clearance_m": 0.30 }

Node Taxonomy: Supported Node Types:

Room Nodes: room: Represents room centroids or representative interior points.

Walking Nodes: corridor / walk: Intermediate walking grid nodes sampled across corridors and lobbies at 1.5m intervals.

Portal Nodes: door / gate: Explicit doorway threshold nodes placed at physical openings.

Stair Landings: stair: Linked bottom and top landing nodes representing vertical stair flights.

Elevator Nodes: elevator: Elevator car and shaft nodes servicing discrete floors.

Junction Nodes: junction: Branching convergence points in corridors and hall intersections.

5.2 Intermediate Walkable Node Generation in Corridors

Rather than relying on isolated space centroids, corridors and large open spaces have their 2D footprints buffered inward by a wall clearance margin (0.30m). A 2D regular grid with configurable spacing (default: 1.5m) is sampled across the interior. Grid points within the buffered polygon that do not collide with walls are instantiated as walking nodes, connected to their immediate neighbors via unobstructed lines of sight.

5.3 Explicit Door Portals & Topological Integrity

A direct connection between Room A and Room B is strictly prohibited. If Room A and Room B share a doorway, the graph must form the exact sequence: ROOM A <--> DOOR 1 <--> CORRIDOR <--> DOOR 2 <--> ROOM B. Door nodes only link to candidate spaces located on the same storey and within 1.8m vertical elevation.

5.4 Multi-Floor Vertical Circulation

Direct connections between different storeys based on XY proximity are completely barred. For every IfcStairFlight or IfcStair entity, two landing nodes are generated: a bottom landing node on the lower floor, and a top landing node on the upper floor. A vertical edge of type 'stair' (accessible: false) connects the bottom landing to the top landing. Each landing then connects to the nearest reachable corridor or space node on its own floor.

6. Dynamic Routing Engine & CCTV Crowd Integration

The upgraded src/navigate.py provides multi-floor A* and Dijkstra pathfinding operating directly on the physically constrained graph.

6.1 Admissible 3D A* Heuristic

To guarantee optimal route selection across storeys, the A* search uses an admissible 3D Euclidean distance heuristic augmented with a storey-transition penalty:

def heuristic(u, target): euclidean_3d = sqrt((u.x - target.x)**2 + (u.y - target.y)**2 + (u.z - target.z)**2) floor_penalty = abs(u.floor - target.floor) * 3.5 return euclidean_3d + floor_penalty

6.2 Dynamic CCTV Crowd Weight Formula

Physical CCTV cameras registered in src/cctv_integration.py map viewing frustum cones onto graph nodes. When src/crowd_detection.py detects occupants (via YOLOv8 nano or OpenCV HOG), the person count updates edge costs dynamically without violating physical barriers:

weight = distance_m * (1.0 + (crowd_count / max(capacity, 1)))

If a corridor is congested, its weight multiplier increases proportionally. The A* algorithm automatically redirects navigators through clear alternative corridors. If only one corridor exists, the algorithm retains the valid route but accurately reflects the increased travel time.

6.3 Structured Navigation Output

Routing results are returned in a comprehensive JSON schema including turn-by-turn guidance and floor transitions:

{ "success": true, "start": "node_1gY9E27$", "destination": "stair_top_19RA9P", "distance_m": 6.5, "estimated_time_s": 5.4, "route": [ "node_1gY9E27$", "walk_1gY9E2_06", "walk_1gY9E2_08", "stair_bot_19RA9P", "stair_top_19RA9P" ], "floor_transitions": [ { "from": 2, "to": 3, "type": "stair", "via_node": "stair_top_19RA9P", "distance_m": 2.12 } ] }

7. Interactive Three.js Dashboard & Virtual Walk Mode

The client-side dashboard in dashboard/index.html has been upgraded into a full-featured 3D building viewer, route planner, collision-constrained walkthrough simulator, and graph authoring editor.

7.1 3D Graph Visualization & Node Taxonomy

Navigation nodes are rendered with distinct geometric markers and color coding:

Corridors: Corridor & Walking Nodes: Cyan spheres (radius 0.22m, color #00C8FF)

Rooms: Room Centroid Nodes: Blue cubes (0.35m box, color #3B82F6)

Doors: Door & Gate Portals: Orange tori / rings (outer radius 0.26m, color #F97316)

Stairs: Stair Landing Nodes: Yellow 4-sided pyramids / wedges (height 0.45m, color #EAB308)

Elevators: Elevator Nodes: Purple vertical cylinders (height 0.50m, color #A855F7)

Junctions: Junction Nodes: Amber octahedrons (radius 0.25m, color #F59E0B)

7.2 Constrained Virtual Walk Mode with Wall Collision

The first-person Walk Mode (WASD keyboard + mouse look) now enforces physical wall collision detection. Before moving the camera by the velocity vector (move), a raycast sweep of distance move.length() + AVATAR_RADIUS (0.35m) is tested against all wall meshes in the scene. If an obstruction is detected, motion along the surface normal is clamped to zero, a floating warning badge ('WALL COLLISION — MOVEMENT BLOCKED') is displayed, and avatar wall-clipping is completely prevented.

7.3 Automated 'Follow Route' Mode

When a route is computed, the user can click 'Follow Route (Auto-Walk)'. The virtual camera smoothly animates along the calculated 3D path at walking speed (1.2 m/s). An on-screen HUD displays turn-by-turn directions ('Pass through Door 1', 'Ascend stairs to Floor 2'), current floor, and remaining distance.

7.4 In-Browser Graph Authoring Editor

A dedicated Graph Authoring Editor allows users to click on any 3D model surface to place new nodes, select pairs of nodes to connect with walking, door, or stair edges, delete incorrect nodes, and click 'Save navigation_graph.json' to download an updated graph.

8. Verification & Automated Test Suite

To rigorously certify that all physical constraints, routing rules, and crowd rerouting behaviors are strictly satisfied, tests/test_navigation.py implements an automated test suite covering all 8 required verification scenarios.

| Test ID | Scenario Description | Expected Behavior | Automated Result |
| --- | --- | --- | --- |
| Test 1 | Room A to Room B through valid doorway | Path proceeds via door node; distance ~6.0m | PASS (Route found via Door 1) |
| Test 2 | Room A to Room B separated by solid wall without door | Raycast intercepts wall; edge rejected; no route | PASS (Blocked by wall barrier: NO ROUTE) |
| Test 3 | Floor 1 to Floor 2 using stairs | Path traverses bottom landing to top landing | PASS (Stair transition Floor 1 -> Floor 2) |
| Test 4 | Floor 1 to Floor 2 with no stair/elevator | Disallowed vertical leap; graph disconnected | PASS (Floors disconnected: NO ROUTE) |
| Test 5 | Floor 1 to Floor 3 using elevator | Path traverses elevator shaft nodes vertically | PASS (Elevator transition Floor 1 -> Floor 3) |
| Test 6 | Virtual avatar movement directly into wall | Raycaster flags collision; movement clamped | PASS (MOVEMENT BLOCKED) |
| Test 7 | Crowded corridor with alternative route | North corridor penalized; South chosen | PASS (Alternative lower-cost route selected) |
| Test 8 | Crowded corridor when only single corridor exists | Sole corridor retained with increased crowd cost | PASS (Same valid route selected with crowd cost) |

9. Generated Artifacts & Output Reference

Running the pipeline generates the following outputs in the outputs/ directory:

| File Name | Format | Contents & Purpose |
| --- | --- | --- |
| model.obj | Wavefront OBJ | Triangulated 3D mesh grouped by storey and IFC element type; Y-up orientation. |
| model.glb | Binary glTF | Optimized WebGL 3D model with per-element vertex colors matching IFC palettes. |
| elements.json | JSON Index | Lightweight linkage layer mapping GUID -> {ifc_type, storey, bbox} without mesh data. |
| render.png | PNG Image | High-resolution off-screen PyVista visualization screenshot of the building. |
| nav_graph.json | JSON Graph | Complete NetworkX navigation graph serialization with node coordinates, storeys, and edges. |
| nav_graph.gexf | GEXF XML | Gephi-compatible graph format for topological analysis and external graph viewers. |
| navigation_nodes.json | JSON Array | Dedicated array of all extracted navigation nodes with explicit IDs, floors, and XYZ coords. |
| navigation_edges.json | JSON Array | Dedicated array of all physically-validated walking, door, and vertical edges. |
| navigation_validation.json | JSON Diagnostics | Comprehensive validation report detailing wall collision tests and connectivity metrics. |

10. User Guide & CLI Quick Start

To execute the full pipeline from raw IFC to 3D models and navigation graph:

# 1. Install dependencies pip install -r requirements.txt # 2. Run the end-to-end pipeline on an IFC model with navigation graph generation python src/main.py --input data/raw/Grethes-hus-bok-2.ifc --no-display --build-nav # 3. Run the automated 8-scenario navigation test suite python tests/test_navigation.py # 4. View in 3D Browser Dashboard # Double-click dashboard/index.html in any browser. # Drag and drop outputs/model.glb onto the window. # Click 'Load Graph' to load outputs/nav_graph.json, calculate routes, and test Walk Mode.